| Exam Automation Technologies Summer 2025 |  |
| --- | --- |
| First Name: | Last Name: |
| Matriculation number: | Study program: |

Instructions:

- You can only take part in the exam if you are registered in the campus management system.

- Prepare your student ID and a photo ID card on your desk.

- Label each exam sheet with your name. Start a new exam sheet for each task.

- Answers must be given with a complete, comprehensible solution. Answers without any context will not be considered. Answers are accepted in German and English.

- Permitted tools are (exclusively): black/blue pens (indelible ink), a triangle/scale, a non-programmable calculator without a graphic display and 1 DIN A4 cheat sheets.

- The exam time is 120 minutes

Evaluation:

| Question | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum score | 14 | 16 | 4 | 4 | 4 | 4 | 4 | 10 | 60 |
| Score secured |  |  |  |  |  |  |  |  |  |

Question 1: Match the types of magnetostrictive effect (A to D) with the measurement/use (1 to 4)

- Wiedemann effect

- Villari effect

- Matteucci effect

- Joule effect

- Stress → changes magnetic properties (force/stress measurement)

- Length/volume change under a magnetic field; basis of magnetostriction and ultrasound transducers

- Used in magnetostrictive position sensors (torsional wave/position)

- Torque measurement (inverse Wiedemann)

Answer:

| Magnetostrictive effect (A to D) | Measurement/use (1 to 4) |
| --- | --- |

Question 2: Given are some scenarios (A-D) where sensor technologies are needed. Also, some options for the sensor principle (1-4) are mentioned. Match the scenario with the best sensor principle.

- Shaft torque in a rotating steel shaft (non-contact preferred)

- Short-stroke (≤5 mm) displacement of a metallic target in a harsh, oily environment

- Detecting current ripple without inserting series resistance

- Long-stroke absolute linear position along a rod (cylinder-type measurement)

- Magnetostrictive time-of-flight position sensing (torsional wave)

- Hall / magnetoresistive magnetic field sensing.

- Inductive sensing (variable reluctance)

- Matteucci/Villari-type magnetoelastic sensing (stress–magnetisation coupling)

Answer:

| Scenario (A to D) | Sensor principle (1 to 4) |
| --- | --- |

Final solution:_____________                                                                                                                            (____/ 2 )